TEST DATE:	08.07.1995

TESTED BY:	L. Sykes, S. Sykes C.N.C.C. Technical Group

TEST TYPE:	Standard

RESULT/REPORT:

All anchors placements prefixed reference 'KMC' were subjected to the standard test. All the anchor placements were in a satisfactory condition.

ACTION:

None. Next test period 1998

TEST DATE:	03.01.1998

TESTED BY:	S. Goodwill C.N.C.C. Technical Group

TEST TYPE:	Standard

RESULT/REPORT:

All anchors placements prefixed reference 'KMC' were subjected to the standard test. All the anchor placements were in a satisfactory condition with the exception of anchor references KMC/35 & KMC/28 which showed slight rotational movement.

ACTION:

Anchor references KMC/35 & KMC /28 to be replaced before 03.1998.

MAINTENANCE DATE:	03.01.1998

MAINTENANCE BY:	S.Goodwill C.N.C.C. Technical Group

MAINTENANCE REPORT:

Anchor reference KMC/35 was removed and replaced using resifix 3+ resin batch number E127 on 03.01.1998.

Anchor reference KMC/28 to replaced within two months, (03.1998).

Anchor reference KMC/42 was placed on 03.01.1998 to replace an existing Petzl long life. Resifix 3+, batch number E127 was used to secure the placement.

MAINTENANCE DATE:	10.01.1998

MAINTENANCE BY:	S.Goodwill C.N.C.C. Technical Group

MAINTENANCE REPORT:

Anchor references KMC/43 & KMC/44 were placed on 10.01.1998 to replace existing Petzl long lifes. Resifix 3+, batch number M277 was used to secure the placement.

MAINTENANCE DATE:	28.02.1998

MAINTENANCE BY:	L. Sykes, G .Jones C.N.C.C. Technical Group

MAINTENANCE REPORT:

Anchor reference KMC/45 was placed using resifix 3+ resin batch number E127 on 28.02.1998. KMC/28 was replaced using Resifix 3+ batch number E127

MAINTENANCE DATE:	23 July 1999

MAINTENANCE BY:	S. Goodwill C.N.C.C. Technical Group

MAINTENANCE REPORT:

Anchor placements refs: KMC/01/ KMC/02, KMC/03 was removed and re-installed using Resifix 3+ batch No. N108.

MAINTENANCE DATE:	10.02.2000

MAINTENANCE BY:	L. Sykes, S. Sykes C.N.C.C. Technical Group

MAINTENANCE REPORT:

Anchor placements refs:KMC/46, KMC/47, KMC/48, KMC/49, KMC/50 were installed using Resifix 3+ batch No. F149, to facilitate rescue.

MAINTENANCE DATE:	21.02.2000

MAINTENANCE BY:	F. Weeks, T. Redfern  CRO

MAINTENANCE REPORT:

Anchor placements refs: KMC/51, KMC/52 were installed using Resifix 3+ batch No. F149, to facilitate rescue.

TEST DATE:	23.06.2005

TESTED BY: L. Sykes. S. Sykes C.N.C.C. Technical Group

TEST TYPE:	Standard

RESULT/REPORT:

All anchors placements prefixed reference 'KMC' were subjected to the standard test. Wire cable was removed and replaced with static rope for rescue.

ACTION:

None, next test period 2009