TEST DATE:	16.08.1993

TESTED BY:	L. Sykes, S. Sykes C.N.C.C. Technical Group

TEST TYPE:	Standard

RESULT/REPORT:

All anchors placements prefixed reference 'RP/PS' were subjected to the standard test. All the anchor placements were in a satisfactory condition.

ACTION:

None. Next test period 1995

TEST DATE:	13.04.1995

TESTED BY:	L. Sykes, S. Sykes C.N.C.C. Technical Group

TEST TYPE:	Standard

RESULT/REPORT:

All anchors placements prefixed reference 'RP/PS' were subjected to the standard test. All the anchor placements were in a satisfactory condition.

ACTION:

None. Next test period 1997

TEST DATE:	12.03.1997

TESTED BY:	L. Sykes, S. Sykes C.N.C.C. Technical Group

TEST TYPE:	Standard

RESULT/REPORT:

All anchors placements prefixed reference 'RP/PS' were subjected to the standard test. All the anchor placements were in a satisfactory condition with the exception of RP/PS/28, RP/PS/31, RP/PS/32 all these anchors have rotational movement in excess of 1mm.

ACTION:

Re-place anchor placements within 28 days.

MAINTENANCE DATE:	30.03.1997

MAINTENANCE BY:	S.Goodwill     C.N.C.C. Technical Group

MAINTENANCE REPORT:

Anchor refs: RP/PS/28, RP/PS/31, RP/PS/32 were removed and replaced with new Eco anchors secured with Resifix 3+ chemical mortar.

MAINTENANCE DATE:	06.04.1997

MAINTENANCE BY:	S. Goodwill    C.N.C.C. Technical Group

MAINTENANCE REPORT:

Anchor refs: RP/PS/ 14 , RP/PS/16   were removed and replaced with new Eco anchors secured with Resifix 3+ chemical mortar.

MAINTENANCE DATE:	03.08.1997

MAINTENANCE BY:	S. Goodwill    C.N.C.C. Technical Group

MAINTENANCE REPORT:

Anchor ref: RP/PS/ 15  was removed and replaced with a new Eco anchor secured with Resifix 3+ chemical mortar batch number E127.

MAINTENANCE DATE:	04.04.1998

MAINTENANCE BY:	S. Goodwill    C.N.C.C. Technical Group

MAINTENANCE REPORT:

Anchor ref: RP/PS/ 22 & RP/PS/24  were removed and replaced with a new Eco anchors secured with Resifix 3+ chemical mortar batch number E127. A new anchor placement RP/PS/29 was installed before the bridge.

MAINTENANCE DATE:	14.06.1998

MAINTENANCE BY:	S. Goodwill    C.N.C.C. Technical Group

MAINTENANCE REPORT:

Anchor ref: RP/PS/ 33, RP/PS/34 & RP/PS/35  were placed and secured with Resifix 3+ chemical mortar batch number C268.

TEST DATE:	21.07.2001

TESTED BY:	L. Sykes, S. Sykes C.N.C.C. Technical Group

TEST TYPE:	Standard

RESULT/REPORT:

All anchors placements prefixed reference 'RP/PS' were subjected to the standard test. All the anchor placements were in a satisfactory condition.

ACTION:

None. Next test period 2004

TEST DATE:	30.04.2004

TESTED BY:	L. Sykes, S. Sykes C.N.C.C. Technical Group

TEST TYPE:	Standard

RESULT/REPORT:

All anchors placements prefixed reference 'RP/PS' were subjected to the standard test. All the anchor placements were in a satisfactory condition.

ACTION:

None. Next test period 2008

TEST DATE:	10.09.2005

TESTED BY:	L. Sykes, S. Sykes C.N.C.C. Technical Group

TEST TYPE:	Standard

RESULT/REPORT:

All anchors placements prefixed reference 'RP/PS' were subjected to the standard test. All the anchor placements were in a satisfactory condition.

ACTION:

None. Next test period 2010